# DNA extraction from a single fly for PCR

### Introduction:

This procedure is designed to isolate genomic DNA for PCR from single flies.

### Materials:

Squishing buffer: 10 mM Tris-Cl pH 8.2, 1 mM EDTA, 25 mM NaCl

Proteinase K:  20 mg/ml stock (Invitrogen), stored in the freezer

### Preparation:

- Put each fly to be tested in a 0.5 microfuge tube and label the tube.

- Thaw the Proteinase K stock and add to a working volume of squishing buffer for 200 µg/ml final Proteinase K.

  - Note:  The squishing buffer can be stored at room temperature, but the Proteinase K must be added fresh each time.  Any excess squishing buffer + Proteinase K must be discarded at the end of the day.

### Procedure:

- Mash each fly for 5 - 10 seconds with a pipette tip containing 50 µl of squishing buffer without depressing the pipette plunger (sufficient liquid escapes from the tip).

- Expel the remaining squishing buffer into fly mixture.

- Incubate at 55°C for ½-1 hr

- Inactivate the Proteinase K by heating to 95 °C for 2-5 minutes. Ensure contact between the tube and the heating block.

### Notes:

- The preparation can be stored at 4 °C for months.

- Typically, 1 µl of the DNA prep is used in a 10-15 µl reaction volume.

- It does not matter if fly parts (wings, bristles, legs) are inadvertently added to the PCR mixture.

- Product will typically start to appear after 24-25 cycles, but 28-30 cycles seems to give maximal yield.

- Increasing the number of flies does not seem to increase the signal significantly, probably due to increasing concentrations of inhibitors.

- There should be no problem scaling up the number of flies screened if the volume is increased proportionately.